Digiview4i Pro

Technical specification

DIGIVIEW4I PRO

Camera Input	4 Channel

Video Input	4 x BNC

Video Output	1VGA (1280x1024) +2 x BNC

Audio Input	1 x RCA

Audio Output	1 x RCA

Operating System	Embedded Linux OS

Internal HDD	Supports 1 SATA Device (HDDx1), upto 2TB

Serial Interface	RS-485

USB	2 x USB connectors

Dimension	250mm x 225mm x 45mm ( L x W x H )

Weight (w/o HDD/DVD±RW)	1.0 Kg

Adaptor Input 	100~240V, Output 12V 5A

Power Source	60 W

Display Speed	Digiview4i Pro - 100 fps

Display Resolution (Pixel)	704 X 576

Recording Compression	H.264 main profile

Network Interface	10/100 Ethernet

Network Protocol	TCP/IP, PPPOE, DDNS, UPNP, FTP and NTP

Network Application	Windows XP/VISTA/7/8 (PC Client system)

Network 3G / GPRS	Windows mobile; iPhone; Blackberry; Symbian;

Android system and 3G

Motion Detection	Detected Frames: Unlimited / Channel

Sensitivity: 6 levels for each channel

Area: 192 (16x12) surveyed area,

Programmable individually

DVR/OSD Control GUI	16-bit True Color

DVR/OSD Control Languages	English, French, Portugal, Russia, Italian, Spanish, Thai, Greek, Japanese, German,

Turkish, Polski , Finnish and Farsi. Chinese (28 Languages)

DVR/OSD Camera Title	alpha numeric

DVR/OSD Remote Controller	IR Remote Control

DVR/OSD System Upgrade	via USB / Network

DVR/OSD Pan/Tilt/Zoom Control	Control From RS485 Interface

CAMERAS

Camera Sensor	1/3.3” Color CMOS

Resolution	700 TVL

Lens	3.6mm Fixed

Effective Pixels	720 x 576

LED Quantity	24pcs

IR Range	Up to 15m

Minimum Illumination	0.01 Lux / F1.2, 0 Lux (IR on)

Gamma	0.45 (AGC off)

Scanning System	2:1 Interlace

Sync System	Interior

Back Light Compensation	ON

White Balance	Auto

Gain Control	Auto

ICR	Removable IR Cut Filter

Video Output	1 Vp-p 75 Ohm,

F-BNC Connector

Input Voltage	DC12V +/- 10%

Input Current 	90mA (IR off)  /  270mA (IR on)

Operating Temperature	-10ºC to +45ºC

Operating Humidity	80% RH