HOW TO WIRE

THE ADE SONADE 2000 SOUNDER/STROB TO THE WSM UNIT.

(TIP)

PROGRAMME THE WSM TO THE  INFINITE PANEL BEFORE FITTING AND CONNECTING TO THE SONADE SOUNDER.

TERMINAL (+)    OF THE WSM UNIT GOES TO TERMINAL (D) OF THE SONADE

(TIP) RED WIRE

TERMINAL (-)     OF THE WSM UNIT GOES TO TERMINAL (T) OF THE SONADE

(TIP) BLACK WIRE

TERMINAL (-T)   OF THE WSM UNIT GOES TO TERMINAL (B) OF THE SONADE

(TIP) BLUE WIRE

TERMINAL (-S)   OF THE WSM UNIT GOES TO TERMINAL (- STROBE) OF THE SONADE

(TIP)  YELLOW WIRE

FIT A WIRE LINK INSIDE THE SONADE FROM TERMINAL (D) TO TERMINAL STROBE (+)

(TIP) RED LINK WIRE

FIT A WIRE LINK INSIDE THE SONADE FROM TERMINAL (E) TO TERMINAL (A)

(TIP)   BLACK LINK WIRE

ONCE ALL WIRES ARE CONNECTED PLUG IN MAINS TRANSFORMER TO THE WSM THE RED LED ON THE SONADE SHOULD FLASH.

CONNECT THE BATTERY IN THE SONADE TO THE TERMINALS MARKED BATT (+) (-) IN THE SONADE.

TEST SOUNDER  IF IT SOUNDS  FIT COVER TO THE SONADE.

NOTE:- THE EXTERNAL SOUNDER TEST WILL ONLY TEST THE SOUNDER AND NOT THE STROBE.

.