Multi-Call

The Multiple Door & Gate Intercom System

1Wire-Free Radio Door Phones for Apartments,

Gated Communities and Offices.

## Exclusive technology from

.

The radio door phone system comprises an outdoor caller unit with an armoured cover, a main controller, a radio relay controller and a completely cordless radio handset.

Connected to the main controller, the outdoor caller unit transmits information to all controllers; the information is then forwarded onto the cordless handset, which is used for voice communication and remote commands to open doors and gates.

There are 2 types of caller unit:

One has a coded access keypad with caller buttons; the other is a standard caller button option.

The keypad model, has an automatic opening key facility that can be programmed to function only at set times (ideal for surgeries, to permit timed deliveries or for tradesmen etc.)

Armoured covers ensure the caller units are highly resistant to vandalism and have an attractive high quality appearance.

Whether you are in a bedroom, bathroom, living room, in the garden or the garage, users can be totally mobile when they answer a call, either choosing to stay where they are or move around thanks to the radio handset. What’s more, you can have as many handsets as the user feels necessary.

Total freedom – Total Control

### Logisty – The leading edge of Innovation

Installer Benefits

Minimal Installation Time

No multi-core cabling or complex programming required.

No need for any heavy work on the premises

This system does not require any cable trenches to be made or holes drilled in the walls of apartments. This means absolutely no damage to the premises.

Ideal for difficult sites

E.g. classified or listed buildings.

No need to go into each apartment

As there is no cabling to be done, the interior handset units can be entrusted to a 3rd person to hand over to the resident.

(e.g. householders’ association, caretaker, etc.).

Everything is prepared for you

- A preparation sheet is provided to help calculate the right number of devices according to site configuration (ref. 803471).

- A radio range tester is provided making it easy to determine the best position for the controllers with respect to the radio range.

Advantages for the installer

- Easy to adapt to the required number of apartments.

- Assembly kit with slides making it easy to fit modules together.

User Benefits

Totally Mobile interior handset unit

Users can move around their home with the interior handset unit.

Check main door access

To prevent unwanted visitors from getting inside.

Check main door closing

By looking at the handset unit’s LCD screen (CP500 position contact to be connected).

Check main gate access

To prevent unwanted visitors from getting inside.

Check main gate closing

By looking at the handset unit’s LCD screen (CP500 position contact to be connected).

Open-ended

Unlimited number of handset units per apartment or per company.

Control several devices using the same remote control

Lighting, timer facility, automatic control system for gate or garage door, main door, automatic barrier etc.

General Technical Data

DynaPass® reliable radio transmission.

Radio range: up to 400 metres in free field.

High fidelity sound.

Fixing elements and accessories provided.

ABS plastic box.

Operating temperature from 0° C to + 50° C.

IP41 protection index: protection against solid bodies > 1 mm

and water drops and condensation.

Adjustable volume and bell, 3 types of bell.

Operation of inside unit in hands free or private conversation

with the handset removed.

Weight: 375 g.

Dimensions: (L x H x D): 145 x 250 x 50 mm.

State of garage door

1 and 2

Overview

Optional programmable functions:

• Sounder with operation of electrical latch.

Electric latch time can be set from 2 to 5 seconds. Ideal for small businesses opening at set times. The keypad caller unit is needed for this feature.

• Common area lighting.

Can be adapted to most configurations. Starting with 7 basic modules, the outdoor caller unit can be easily assembled and installed for buildings with 2 to 30 apartments.

Failsafe radio communication

Like all Logisty equipment, the mobile doorphone ensures ultra-reliable radio transmission. Patented under the name DynaPass®, this innovative technology protects radio communication against interference, providing an optimum link between the outdoor caller unit and the handset.

Dynamic radio technology

Each interior handset and controller transmitter/receiver keeps a constant check on the most available radio channels.

At the start of communication, the system selects a free radio channel.

If there is any interference on this channel during communication, the self-synchronising system automatically switches to another available channel.

What’s more, the radio doorphone is based on a specific patented radio technology.

Unlimited number of handsets per apartment

As many handsets as you wish can be installed.

This is very useful for large dwellings or those with outbuildings.

High quality sound

Thanks to the DynaPass® technology the system’s listen-in quality is extremely good, both in terms of sound perfection and level, whatever the type of communication chosen (in both hands-free and hand-held mode).

Radio range

Depending on environmental and installation conditions, optimum communication between the caller unit and each interior handset is obtained by including the right number of radio controllers in each installation (7 maximum).

On average 1 radio controller covers 6 dwellings.

It is advisable to test the radio range using the RADIOTEST kit.

The technology

With all their technical requirements and the disruption to property, with the additional disturbance they cause during installation to residents, many intercom systems prove to be more trouble than they are worth.

With our patented doorphone technology, there is no need for any wiring between the outdoor caller unit and the handsets.

Only Logisty offers such innovation.

1

2

SYSTEM DATA:

• DynaPass® failsafe radio transmission.

• Specific radio technology for building

doorphones.

• Radio range: depending on environmental and installation conditions, radio coverage for optimum communication between the outdoor caller unit and each interior handset is obtained by placing a suitable number of radio controllers in each installation (7 maximum). On average, one radio controller is suitable for 6 apartments.

• High audio quality.

• Simple radio programming.

• Backup batteries providing 2-day battery life in the event of a power failure.

• Guarantee: 2 + 1 years.

• 24 V AC power: supplied by standard radio controller.

EXTERNAL CALLER UNIT:

COMMON DATA:

• Armoured metal cover.

• DynaPass® failsafe radio transmission.

• Electronic module polycarbonate housing.

• Built-in listen-in microphone and loud speaker.

• Caller unit with or without keypad.

• Comes with permanently back-lit “name” tags, easily modifiable.

• Name tag holder included.

• Modules with armoured cover including the following:

Loudspeaker module / 2-button top module / button-free middle module / 4-button middle module / Keypad middle module / Button-free top/bottom module / 2-button bottom module.

• Several layouts possible (with keys opposite or not, modules side by side or on top of one another).

• 10-metre cable supplied with each electronic module.

• Operating T°: -20°C to + 50°C.

• Degree of protection: IP54 against dust and water spray.

• Dimension of one module with armoured

cover: 145 x 232.5 x 15.5 mm (W x H x D).

DATA SPECIFIC TO CALLER UNIT WITHOUT KEYPAD:

• From 2 to 30 buttons maximum.

DATA SPECIFIC TO CALLER UNIT WITH KEYPAD:

• From 2 to 26 buttons maximum.

• Back-lit keys on keypad.

• Latch and automatic opening control keys.

MAIN CONTROLLER:

• Controls and supplies power to all types of standard 12 V latches (1.5 A max.).

• Controls electrical latches or electromagnetic closers with external power supply (operating mechanism to be connected to a16 V rms max. / 22.6 V max peak/30 V DC max. circuit).

• Input terminals for connection of a dry-type door position contact (normally closed) (to display door/gate status on handsets).

• Automatic opening device control.

• Setting of the latch control time

(2 or 5 seconds).

• Setting of the automatic opening device time (from 0.5 to 2.5 sec.).

• Backup battery for 2-day battery life in the even of a mains power failure.

• Operating T°: -20°C to + 50°C.

• Degree of protection: IP54 against dust and water spray.

• Dimension: 160 x 230 x 66 mm (W x H x D).

STANDARD RADIO CONTROLLER

AND EXTENDED RADIO CONTROLLER:

COMMON DATA:

• DynaPass® failsafe radio transmission.

• Telescopic antenna (length extended 780 mm).

• Operating T°: -20°C to + 50°C.

• Degree of protection: IP54 against dust and water spray.

• Dimensions of controller: 160 x 230 x 66 mm (W x H x D).

• Backup battery for 2-day battery life in the event of a mains power failure.

• Polycarbonate enclosure.

DATA SPECIFIC TO THE STANDARD RADIO CONTROLLER:

• Controls lighting (N.O. output to control remote control type lighting, operating mechanism to be connected to a 16 V rms max. 22.6 V max peak / 30 V DC max. circuit).

• Automatic activation of lighting possible on detection of door, gate and latch opening.

• Clock input to programme automatic opening and latch keys for direct access (with keypad caller unit).

• 24 V AC power supply via external transformer, mounted on DIN rail.

DATA SPECIFIC TO EXTENDED RADIO CONTROLLER:

• For adapting the radio range needed for the site.

Up to 6 extended radio controllers can be added.

INTERIOR HANDSETS:

COMMON DATA:

• DynaPass® failsafe radio transmission.

• ABS casing.

• LCD screen.

• Handsfree or hand-held operation, without audio distortion.

• Handset completely recharged in 24 hours.

• Unlimited number of handsets per apartment.

• Adjustment of volume (4 levels) and type of ring

(3 tones).

• Indication of radio reception level.

• Indication of battery life.

• Degree of protection: IP 41 against dust and solid matter < 1 mm and vertical droplets.

DATA RELATING TO HANDSET WITH BATTERY-OPERATED

BASE: (L5141 & L5142)

• Li/ion battery in handset.

• Handset battery life when off base: 4 days (in normal conditions of use).

• Comes with 4 LR20 alkaline batteries for the base.

• Base battery life: 4 years

(in normal conditions of use).

• Base fixed to wall (fixing device supplied) or placed on a flat surface (retractable legs under base).

• Dim. of controller alone: 60 x 255 x 30 mm (W x H x D).

- 10 x 30 s. communications a day and 10 latch controls.

• Dim. of battery-operated base:

250 x 145 x 50 mm (W x H x D).

• Operating T°: from 0°C to 50°C.

DATA RELATING TO HANDSET WITH MAINS-OPERATED BASE: (L5130 & L5131)

• Handset comes with Ni-Cad battery.•

• Battery life of handset when off base: 2 days (in normal conditions of use)**.

• Wall-mounted support with transformer supplied.

• Dim.: (W x H x D): 100 x 125 x 140 mm.

• Operating T°: from 0°C to 50°C.

DATA RELATING TO SINGLE FUNCTION DOORPHONE

HANDSET: (L5130)

• Can control 1 latch.

• Displays main door and automatic opening device status (if position contact connected).

DATA RELATING TO 3-FUNCTION DOORPHONE

HANDSET: (L5131)

• Can control 1 latch. (e.g. 12v DC Door Latch)

• Can control 1 automatic opening device (e.g. powered gate)

• Can control 1 lighting device (when not communicating).

• Displays main door and automatic opening device status (if position contact connected).

DATA RELATING TO 7-FUNCTION HANDSET: (L5142)

• Can communicate with 2 different entrances or gates.

• Can control 2 latches.

• Can control 2 automatic opening devices. (e.g. powered gates)

• Can control 1 lighting device (when not communicating).

• Displays the opening status of main doors and opening devices (if position contact connected).